- Disable antivirus software.

- Plug one end of a USB-C or USB-C to A cable into the PD input port of the device, and connect the other end into the computer. Connect the USB-C cable that is integrated with the product into the computer.

- Launch the firmware update tool exe.

- Press any key on the keyboard to continue.

- When the firmware tool opens, it shows the current FW version. (If it does not, try flipping the USB-C cable you connected to the PD port 180 degrees.)

- Select the folder in upper left and select the firmware update file (.bin) to update.

- In the Configuration panel, go to the interface and change UART to USB.

- Then you should be able to hit play to download the new firmware to the device.

- Confirm flash start by clicking OK.

- Wait for the firmware flash to finish.

- Unplug and replug the cables and ensure that the new firmware was flashed.